POSITIVE ENERGY DISTRICTS: A CASE STUDY IN THE MEDITERRANEAN AREA

Francesco Guarino, Roberta Rincione1, Luisa F. Cabeza, Gemma Gasa2, Carles Mateu2, Maurizio Cellura1.

Keywords: Positive energy district; Sustainability assessment; Life Cycle Assessment; building simulation

The high impact of the built environment on the decarbonization of the energy sector is pushing towards the evolution of the zero energy building concept into the  Positive Energy Districts (PED) one, as declined by JPI Urban Europe and in the SETPLAN as energy flexible urban areas with annual net zero energy import that drive towards the achievement of carbon neutrality. In this context, the objective of the research is the analysis of the potential  to achieve the status of Positive Energy District of a neighborhood located in Balaguer (Lleida, Spain), and the assessment of environmental impacts of the redesign solutions required to reach it. The district is composed of five different non-residential buildings, with a total area of 3,354.44 m2. Data acquisition was performed for building characteristics, occupancy information, and energy uses within the buildings. The geometrical modeling was performed in SketchUp environment, and energy simulation and calibration was performed in EnergyPlus.

Different retrofit scenarios to achieve high energy performances were investigated at the district level also including on-site generation of energy from renewable sources, their environmental impacts investigated through the use of the Life Cycle Assessment methodology.

The results show that the PED target can be achieved as well as a significant reduction in consumption compared to the initial scenario through simple redesign solutions of around 30% of electrical consumption and roughly 40% of the thermal needs. The overall life cycle  CO2eq is equal to 16.6 kg CO2eq/(m2·y) in the initial scenario and 9.4 kg CO2eq/(m2·y) in the redesign scenario. Also a significant reduction 43.75% in carbon uses is achievable where the increase of embodied carbon is offset by the significant reduction in use stage energy uses. An analysis of the state of the buildings and the subsequent interventions show that reaching the PED target is the right direction for achieving significant carbon reduction and better energy planning, where achieving zero balance is easier with more actors helping to mediate the difference at the individual building level.

University of Palermo, Viale delle scienze, Building 9, 90128 Palermo (Italy), francesco.guarino@unipa.it

GREiA Research Group, Universitat de Lleida, Pere de Cabrera s/n, 25001 Lleida, Spain, luisaf.cabeza@udl.cat