The impact of the horizon in evaluating the potential of solar power

Luigi Bottecchia,,*, Mattia Dallapiccola1, Lukas Kranzl2 and Pietro Zambelli1

Keywords: solar; photovoltaics; potential; gis; horizon height

In the current energy transition, solar energy is a key source to achieve the decarbonization of the energy system and the whole society. In this sense, Geographic Information System (GIS) tools are crucial to allow a more precise evaluation of complex environments such as urban areas or mountain locations. Indeed, buildings in the proximity of the analyzed area as well as mountains generate shadows that can affect the solar potential.

Thus, this work analyzes the impact of the horizon height of the surrounding areas (e.g. close by buildings) as well of the more distant obstacles (e.g. mountains) in evaluating the solar potential in a given location. This is possible thanks to a higher spatial resolution of input data. In particular, the analysis focuses on investigating the differences in terms of solar potential between data extrapolated from existing tools and more spatially explicit data that includes horizon height information. The analysis considered both the weather station of Bolzano and a building in Bolzano characterized by complex orography. Firstly, the horizon height was calculated for both locations. Secondly, its effects on the solar potential were evaluated by pointing out in which hours the two locations were under shading. The analysis showed that in locations characterized by a complex orography considering the horizon height results in a consistent difference. Indeed, the total yearly irradiance was reduced by roughly 9%. Additionally, these discrepancies also result in a different hourly behavior that can lead to consistent differences in the utilization of solar power for covering the demand.

Moreover, these results showed that in the early stage of planning traditional tools to evaluate solar potential can still be precise, particularly at a yearly level. However, to carry out more precise analyses, including the horizon height on the calculation can lead to more accurate results and better sizing of the system.

Institute of Renewable Energy, Eurac Research, Via A.Volta 13/A, 39100 Bolzano, Italy

*  corresponding author: luigi.bottecchia@eurac.edu

Energy Economics Group, Institute of Energy Systems and Electrical Drives, Technische Universität Wien, Gusshausstraße 25-29/370-3, 1040 Vienna, Austria