Open data for energy knowledge

Marianna Pozzi, Fabrizio Fattori, Alice Dénarié2, Mario Motta2

Keywords: open data; heating energy consumption; spatial distribution; energy development

- The knowledge related to the consultation of several databases is the basis for greater awareness of the buildings stock and possible changes towards decarbonization of the energy system. A detailed mapping tool is a useful input to identify interesting areas for a decarbonization process. Starting from the Italian Buildings Census, ISTAT, the Energy Performance Certifications (EPC), and Thermal System Cadaster (CURIT), a tool for the estimation of heating demand of residential and tertiary buildings is developed. This tool considers type of buildings, construction ages, climate, and subsectors. The analysis can be improved if other data were available with a great detail. Matching the obtained correlations from EPC with the corresponding number of houses or the corresponding surface of tertiary buildings, the spatial distribution of civil heating demand in each census cell is estimated; the total civil demand is 67.3 TWh/y, divided into 50.9 TWh/y residential and 16.4 TWh/y tertiary. The strength of this mapping tool is the possibility to develop different decarbonization scenarios identifying the more efficient and less emissive technologies, starting from the current systems information obtained from CURIT (e.g. heating systems’ centralization and fuels), and the envelope refurbishment in each cell considering the different needs of buildings and the different available energy sources. The tool is applied to analyze the retrofit in each census cell: a refurbishment about the opaque and/or the transparent envelope of buildings according to the technicians’ suggestions in the EPC database achieving a total reduction in term of energy demand equal to 37.5% (37% residential and 40% tertiary sector). The future of the energy system is evaluated through an optimization model using this heating distribution as input and identifying the optimal configuration of energy system. The results of different future decarbonization scenarios can help cities, regions or specific areas for energy development or planning. This is the reason of the importance to make different type of information about buildings accessible.

Department of Energy, Politecnico di Milano, via Lambruschini 4 20156 Milano Italy, marianna.pozzi@polimi.it

Department of Energy, Politecnico di Milano